SBTSR v0.20 beta - JVFAX with Sound Blaster

-------------------------------------------

Written by 4Z5BS

08/12/96

-------------------------------------------

Overview

========

SBTSR allows you to receive and transmit FM-FAX, SSTV and AM-FAX

using only your Sound Blaster card ! All you need is a

Sound Blaster Pro and some wires to connect the card to your

transceiver.

FM-FAX (or just FAX) and SSTV can be found on all the HF bands from

amateur radio stations and from broadcasters. AM-FAX

is transmitted by the NOAA weather satellites (137 Mhz) and GOES

geostationary weather satellites (1691 Mhz). It is also a great

mode to transmit pictures on the VHF and UHF bands using

FM transmissions.

Hardware requirements

=====================

1. 80486DX2 or better.

2. Sound blaster Pro or better.

Installing

==========

The first thing you have to do is install JVFAX 7.0 or better.

It comes with good documentation so I assume you already

did it. Before you load JVFAX load my program, SBTSR.

After quiting JVFAX you can unload SBTSR from memory using

"SBTSR -u". I wrote a batch file that does that for me:

RUN.BAT

-=-=-=-

SBTSR

JVFAX

SBTSR -u

You might want to add some command to control the Sound Blaster

mixer. Setting the input volume to high will seriously

damage the quality of picture received. When receiving

AM-FAX (from NOAA satellites usually) the AGC circuit

in the sound blaster should be turned off if possible.

Other parameters

================

You can transmit AM-FAX by using the "-a" parameter.

For reception in AM-FAX, in FAX mode, choose a mode

that uses AM or press D to select AM1.

Use -c24 or any other number if the picture is slanted.

This should correct the error in the clock.

Although I believe the program is already calibrated,

cards can have differences. If you see pictures slanted

begin either increasing or decreasing this value.

Good numbers to begin with are -c100 and -c-100.

If you want the program to automatically push PTT

use the -k3f8 parameter if you use COM1. For COM2

use -k2f8. The RTS line in the serial line is on whenever

transmitting. Connecting this line to the P.T.T. of the

transmitter using a simple interface like a transistor

saves you the need to press PTT.

The sound blaster settings are taken from the environment

settings so you don't have to change them usually.

Known Problems

==============

The clock sometimes runs too fast. It seems to be

necessary in SSTV mode; if your clock does not

run too fast in SSTV mode, try to change to WR120

mode and press the '5' key several times.

The Sound Blaster 16-bit card allows much better results

in AM mode. It is also more accurate so if you have

one, use it.

Disclaimer

==========

I shall not be liable for any direct, indirect, consequential or incidential

damages arising out of the use of this software. Use only at your own

risk.

Copyright notes

===============

This program is neither shareware nor public domain.

You have the right to use this program for non commercial purposes without

further permission. You can copy it and share it as long as you

don't charge any money for it.

Sound-Blaster (c) 1989-1991 Creative Labs Inc

JVFAX is the Copyright property of Eberhard Backeshoff, DK8JV.

(c) 1994 by Eberhard Backeshoff, Obschwarzbach 40a, D-40822 Mettmann, Germany.

Thanks

======

Thanks to Ethan Brodsky for the Sound Blaster library

and documentation.

4Z5BS - Oded Regev

PACKET: 4Z5BS@4Z4AAA.ISR.MDLE

EMAIL : 4z5bs@4z4aaa.ampr.org