# NCD.EXE User Manual

NCD.EXE for dos is a Dos program. It can run in MS DOS 6.0 and later. It can be run stand alone, or in a batch file, or be call in other program. It is small, quick and no installation needed. It supports most of the devices of NCD. It can be used to test the device, or do the simple work with NCD device.

The command line option of the NCD.exe is same as that for windows version.

Usage

NCD.exe {- h [topic]}|-v|{ [-p COMPort][-b BaudRate] -d DeviceName function parameter}

-h				Show help of NCD.exe,

Topic		To get the help info of specific device, use device name as topic

-v 				Show version of NCD.exe

-p	COMPort		Set the COM Port, the default port is COM1, the available values are

COM1 to COM 9 for windows 98 and NT.

COM1 to COM 255 for windows 2000/XP/2003

-b	Baudrate		Set the Baudrate of the COM Port, the default value is 9600. the available

values are

1200

2400

4800

9600

19200

38400

-d	DeviceName function parameter

Set the device, function and parameters for that function

DeviceName		name of device, the supported devices are listed as below.

ASEL

8S2S

AVS4X1

E3C

IOADR8X

IOADR16X

IOCONTROLLER

SERVOCONTROLLER

MSERVO

R1X

PWM8X

R16X

R2X

R32X

R4X

R8XPRO

SSR8X

Function 		name of function need to carry out.

parameter			parameter needed for carry out the function

Function name and parameter list can be get by using

c:\>ncd.exe –h devicename

For example, to get the function list of R8X, please input the following line in command window.

c:\>ncd.exe –h R8X	<RETURN>

It will output the following thing.

R8XPRO  Function List

Relay_On            X    X: 1-4,8

Relay_Off           	X    X: 1-4,8

GetRelayStatus      	X    X: 1-4,8

GetAllRelayStatus

StoreDefault

GetDefault

TurnAllRelayOff

TurnAllRelayOn

InvertAllRelay

ReverseRelayOrder

Test2Way

SetAllRelays        	X    X: 0-255

StorePatternInBank  	X    X: 0-15

RecallPatternInBank 	X    X: 0-15

RelaySelect         	X    X: 1-4,8

RelayDeselect       	X    X: 1-4,8

ToggleRelay        	X    X: 1-4,8

SetRelayTimer       	X1 X2 [X3]

X1 : 0-32000, X2 : 0-7, [X3 : 0 or 1]

SetMultiRelaysTimer 	X1 X2 X3 [X4]

X1 : 0-32000, X2-X3 : 0-255   [X4 : 0 or 1]

ReportModeOn

ReportModeOff

ProgramEMUDevNum    X    X: 0-15

Examples

All examples of this document suppose that you have copy the ncd.exe to root dir of C disk. Otherwise, you need to change the current dir to ncd.exe’s dir.

A	Turn on the relay of R1X connected on COM1, with a baudrate of 9600

C:\> ncd.exe –d R1X relay_on

B 	Turn off the relay of R2X connected on COM2, with a baudrate of 4800

C:\> ncd.exe –p COM2 –b 4800 –d R2X relay_off 2

C	Show the function list of R8XPRO

C:\>ncd.exe –h R8XPRO

D	Show the NCD.exe ‘s version

C:\>ncd.exe –v

## Device List

# ASEL  Function List

SetOutput1  In1       	  	In1: 1-16

SetOutput2  In1       		In1: 1-16

SetBothOutput  In1 In2   		In1,In2 : 1-16

SetOutput1    In1 			In1: 1-16

Routes output1 to a given input. This command requires a parameter byte used to select which input (1-16) is routed to output 1.

Example

Input5 is routed to output1.

c:\>NCD.exe  -d  ASEL  SetOutput1   5

SetOutput2    In1 			In1: 1-16

Routes output 2 to a given input. This command requires a parameter used to select which input (1-16) is routed to out-put 2.

Example

Input12 is routed to output2.

c:\>NCD.exe  -d  ASEL  SetOutput2   12

SetBothOutput    In1  In2 		In1,In2 : 1-16

Set the route of both channels at one time. This command requires two parameter bytes used to select which input (1-16) is routed to each output. The first parameter byte routes output 1 to a selected input (1-16). The second parameter byte routes output 2 to a selected input (1-16).

Example

Input8 is routed to output1, and input15 is routed to output2.

c:\>NCD.exe  -d  ASEL  SetBothOutput   8   15

# 8S2S  Function List

SetOutput1  In1   	    	   In1: 1-16

SetOutput2  In1    		   In1: 1-16

SetBothOutput  In1 In2   	   In1,In2 : 1-16

The usage of these three functions is same as that of ASEL.

Example

Input5 is routed to output1.

c:\>NCD.exe  -d  8S2S  SetOutput1   5

Input12 is routed to output2.

c:\>NCD.exe  -d  8S2S  SetOutput2   12

Input8 is routed to output1, and input15 is routed to output2.

c:\>NCD.exe  -d  8S2S  SetBothOutput   8   15

# AVS4X1  Function List

SelectInput    In1  		     In1: 1-4

GetInput

StoreSelectedInput

GetStartupInput

SelectInput     In1   			    In1: 1-4

This command is used to change the stereo A/V input. This command requires a parameter from 1 to 4 indicating desired input selection.

Example

Select 3 as input.

c:\>NCD.exe  -d  AVS4X1  SelectInput   3

GetInput

This command is used to ask the AVS4x1 which input is currently active. This command will return a parameter from 1 to 4 indicating the active input.

Example

Get the active input.

c:\>NCD.exe  -d  AVS4X1  GetInput

StoreSelectedInput

This command stores the currently active input as the default input. The next time power is applied to the AVS4x1, the selected input will automatically be routed to the output.

Example

Store selected input.

c:\>NCD.exe  -d  AVS4X1  StoreSelectedInput

GetStartupInput

This command reports the power-up default input selection. This command will return a value from 1 to 4 indicating the input channel that will become active when power is first applied to the AVS4x1.

Example

Get Startup Input Selection.

c:\>NCD.exe  -d  AVS4X1  GetStartupInput

# E3C  Function List

ProgramDeviceNumber    In1   	    In1: 0-255

EnableAllDevicesExcept   In1      	    In1: 0-255

DisableAllDevicesExcept  In1       		In1: 0-255

DisableSpecificDevice   In1            In1: 0-255

EnableSpecificDevice    In1    	    In1: 0-255

DisableAllDevices

EnableAllDevices

RecallDeviceNumber

ProgramDeviceNumber	  In1			 In1: 0-255

Programs the device number into the controller. Only ONE controller should be attached to your computer at the time of programming. Once programmed, use the E3C commands to control which devices you are speaking to.

Example

Programs the device number 3 into the controller

c:\>NCD.exe  -d  E3C  ProgramDeviceNumber   3

EnableAllDevicesExcept   In1     	  	In1: 0-255

Tells a specific device to ignore your commands, all others will listen.

Example

Disable device 16 only.

c:\>NCD.exe  -d  E3C  EnableAllDevicesExcept   16

DisableAllDevicesExcept   In1       	In1: 0-255

Tells a specific device to listen to your commands, all other devices will ignore your commands.

Example

Enable device 20 only.

c:\>NCD.exe  -d  E3C  DisableAllDevicesExcept   20

DisableSpecificDevice   In1      		 In1: 0-255

Tells a specific device to ignore your commands.

Example

Disable device 11.

c:\>NCD.exe  -d  E3C  DisableSpecificDevice   11

EnableSpecificDevice   In1     		  In1: 0-255

Tells a specific device to listen to your commands.

Example

Enable device 34.

c:\>NCD.exe  -d  E3C  EnableSpecificDevice   34

DisableAllDevices

Tells all devices to ignore your commands.

Example

Disable All Devices.

c:\>NCD.exe  -d  E3C  DisableAllDevices

EnableAllDevices

Tells all devices to respond to your commands.

Example

Enable All Device.

c:\>NCD.exe  -d  E3C  EnableAllDevices

RecallDeviceNumber

Allows you to read the stored device number from the controller.

Example

Read the device's number

c:\>NCD.exe  -d  E3C  RecallDeviceNumber

# IOADR8X  Function List

Test2Way

SetDefault

GetAnalog8   In1    		In1 : 0-4

GetAnalog10  In1   	 	In1 : 0-4

Test2Way

This method can be used to test 2-way communication between the host computer and the relay controller.

Example

Test 2-way communication, true for 2-way communication.

C:\> ncd.exe  –d  IOADR8X  Test2Way

SetDefault

This command can be used to restore all configuration parameters to factory default settings. DO NOT ISSUE THIS COMMAND IF MORE THAN ONE HYBRID OR IOAD168 CONTROLLER IS ATTACHED TO THE SAME SERIAL PORT OR ALL CONTROLLERS WILL BE PROGRAMMED WITH FACTORY DEFAULT VALUES.

Example

Restore all configuration to factory default setting.

C:\> ncd.exe  –d  IOADR8X  SetDefault

GetAnalog8   In1  	  In1 : 0-4

This command requests the analog data value from the user-specified A/D input, and reports back an analog value of 0-255.

Example

Get 8-bits analog value of channel 2.

C:\> ncd.exe  –d  IOADR8X  GetAnalog8  2

GetAnalog10  In1   	 In1 : 0-4

This command requests the analog data value from the user-specified A/D input, and reports back an analog value of 0-255.

Example

Get 10-bits analog value of channel 3.

C:\> ncd.exe  –d  IOADR8X  GetAnalog10  3

SUB DEVICE LIST:

LCD

PORTA

PORTB

PORTC

RELAYBANK

KEYBOARD

SUB_LCD  Function List

Init

SendData    In1		    		In1 : 0-255

SendCommand   In1 		    In1 : 0-255

SendBuffer   In1 				In1 : Text

BacklightOff

BacklightOn

SetPort   In1 			    In1 : 1-3

ClearScreen

HomeCursor

BlankDisplay

ToggleCursorVisible

UnderlineCursor

BlockCursor

MoveCursorLeft

MoveCursorRight

SetPointerPosition   In1    	In1 : 0-63

SetCursorPosition   In1     	In1 : 0-127

Init

This command is used to send an initialization sequence to the character display. While this command was written specifically to initialize a 16x2 display, this command is compatible with all displays we tested. When this command has finished, a flashing cursor should appear in the upper left corner of the display screen. This command sends ASCII character code 85 back to

the user indicating the command has finished execution.

Example

Initial the LCD.

C:\> ncd.exe  –d  IOADR8X.LCD  Init

SendData    In1    			In1 : 0-255

This command is used to send data directly to the display hardware. This command can be used to display text on the screen, however, it is much more efficient to use the LCD Text Buffer command. This command is not used in most applications, but it is provided so just incase you need to adjust some display parameters. This command requires a parameter from 0-255 indicating the data that it is to be sent to the display. This command sends ASCII character code 85 back to the user indicating the command has finished execution.

Example

Send 125 to LCD hardware.

C:\> ncd.exe  –d  IOADR8X.LCD  SendData  125

SendCommand    In1    		In1 : 0-255

This command is used to send a command directly to the display hardware. This command should be used to issue all commands native to the character display. Such commands allow you to be hide and position the cursor, program your own fonts, and clear the display. This command will be demonstrated on the following pages. This command requires a parameter from 0-255 indicating the command that it is to be sent to the display. Misuse of this command can cause unusual and erratic behavior. A complete set of commands is found in your LCD data sheet. Only the most common commands will be discussed in this manual. This command sends ASCII character code 85 back to the user indicating the command has finished execution.

Example

Send Command to display hardware 12.

C:\> ncd.exe  –d  IOADR8X.LCD  SendCommand  12

SendBuffer    In1   		In1 : Text

Once text data has been written to the display, this command will send ASCII character code 85 back to the user indicating completion of this command.

Example

Send "Hello" to buffer.

C:\> ncd.exe  –d  IOADR8X.LCD  SendBuffer  “Hello”

BacklightOff

Turn off the backlight LED. Some displays need to be reinitialized after this command has finished execution. By default, the backlight LED is Off.

Example

Turn off the backlight of LCD.

C:\> ncd.exe  –d  IOADR8X.LCD  BacklightOff

BacklightOn

Turns on the backlight LED. Some displays need to be reinitialized after this command has finished execution.

Example

Turn on the backlight of LCD.

C:\> ncd.exe  –d  IOADR8X.LCD  BacklightOn

SetPort     In1   			 In1 : 1-3

This command is used to route all subsequent LCD commands to the selected port. By default, LCD commands are sent to Port C on Hybrid 8-Relay Controllers. On Hybrid 16-Relay Controllers, all LCD commands are directed to Port A.

Example

Set the LCD's port to 2.

C:\> ncd.exe  –d  IOADR8X.LCD  SetPort  2

ClearScreen

Clear Screen.

Example

C:\> ncd.exe  –d  IOADR8X.LCD  ClearScreen

HomeCursor

Move cursor to top/left character position.

Example

C:\> ncd.exe  –d  IOADR8X.LCD  HomeCursor

BlankDisplay

Blank the display (without clearing).

Example

C:\> ncd.exe  –d  IOADR8X.LCD  BlankDisplay

ToggleCursorVisible

Toggle cursor visible status.

Example

C:\> ncd.exe  –d  IOADR8X.LCD  ToggleCursorVisible

UnderlineCursor

Turn on visible underline cursor.

Example

C:\> ncd.exe  –d  IOADR8X.LCD  UnderlineCursor

BlockCursor

Turn on visible blinking-block cursor.

Example

C:\> ncd.exe  –d  IOADR8X.LCD  BlockCursor

MoveCursorLeft

Move cursor one character left.

Example

C:\> ncd.exe  –d  IOADR8X.LCD  MoveCursorLeft

MoveCursorRight

Move cursor one character right.

Example

C:\> ncd.exe  –d  IOADR8X.LCD  MoveCursorRight

SetPointerPosition   In1  		  In1 : 0-63

Set pointer in character-generator RAM (CG RAM address).

Example

Set pointer in character-generator RAM to 42.

C:\> ncd.exe  –d  IOADR8X.LCD  SetPointerPosition  42

SetCursorPosition    In1  		  In1 : 0-127

Set cursor position (DDRAM address).

Example

Set pointer in character-generator RAM to 115.

C:\> ncd.exe  –d  IOADR8X.LCD  SetCursorPosition  115

# SUB_PORTA  Function List

GetBitsStatus  In1		        In1 : 0-7

PutByte  In1  	    		In1 : 0-255

TurnOnPortBits  In1 		    In1 : 0-7

TurnOffPortBits  In1          In1 : 0-7

ReadPortByte

PAR24  In1  In2             In1 : 1-6  In2 : 0-255

Ad1216  In1  In2            In1 : 0-2  In2 : 0-15

GetBitsStatus  In1 			       In1 : 0-7

This command is used to read the status of data bits on each of the data port. This command returns an ASCII character code 0 or 1 indicating bit status.

Example

Get the third bit status of PortA.

C:\> ncd.exe  –d  IOADR8X.PORTA  GetBitsStatus  2

PutByte   In1			        In1 : 0-255

This command is used to send a byte of data directly to the Port A, B or C of data port on the Hybrid. All lines on Port become an output when this command is issued. This command is very similar to a serial-to-parallel converter. The parameter of this command, 0-255, is written directly to the port and the on/off status of each of the 8 lines appears in the equivalent binary pattern of the parameter.

Example

Put 27 to port A.

C:\> ncd.exe  –d  IOADR8X.PORTA  PutByte  27

TurnOnPortBits     In1        In1 : 0-7

This command is used to set the on status of individual I/O lines on port.

Example

Set the bits 5 of PortA on.

C:\> ncd.exe  –d  IOADR8X.PORTA  TurnOnPortBits  5

TurnOffPortBits    In1        In1 : 0-7

This command is used to set the off status of individual I/O lines on port.

Example

Set the bits 7 of PortA off.

C:\> ncd.exe  –d  IOADR8X.PORTA  TurnOffPortBits  7

ReadPortByte

This command is used to read a binary value from Port , acting like a parallel-to-serial encoder.

Example

Read a binary value from portA.

C:\> ncd.exe  –d  IOADR8X.PORTA  ReadPortByte

PAR24   In1  In2   			 In1 : 1-6  In2 : 0-255

This command sends a byte of data (0-255) to one of the PAR24 output channels (1-6) on Port. The data appears as binary/ parallel data on the 8-bit output of the PAR24.

Example

Put value 66 to channel 4 of PortA.

C:\> ncd.exe  –d  IOADR8X.PORTA  PAR24  4  66

Ad1216   In1  In2    		In1 : 0-2  In2 : 0-15

This command returns a 12-Bit analog value from Channel 0-15, Defined as Device 0-2 on a AD1216 Connected to Port A or B.

Example

Read the analog value from 8 channel, 1 device on PortA.

C:\> ncd.exe  –d  IOADR8X.PORTA  Ad1216  1  8

# SUB_PORTB  Function List

GetBitsStatus   In1   		    In1 : 0-7

PutByte   In1  		        In1 : 0-255

TurnOnPortBits  In1  	        In1 : 0-7

TurnOffPortBits  In1          In1 : 0-7

ReadPortByte

PAR24   In1  In2 	   	    In1 : 1-6  In2 : 0-255

Ad1216  In1  In2            In1 : 0-2  In2 : 0-15

The usage of above functions is same as that of PORTA.

Example

C:\> ncd.exe  –d  IOADR8X.PORTB  GetBitsStatus  3

C:\> ncd.exe  –d  IOADR8X.PORTB  PutByte  72

# SUB_PORTC  Function List

GetBitsStatus   In1 	        In1 : 0-7

PutByte   In1        		In1 : 0-255

TurnOnPortBits   In1  	    In1 : 0-7

TurnOffPortBits   In1         In1 : 0-7

ReadPortByte

The usage of above functions is same as that of PORTA.

Example

C:\> ncd.exe  –d  IOADR8X.PORTC GetBitsStatus   6

C:\> ncd.exe  –d  IOADR8X.PORTC  PutByte     58

# SUB_RELAYBANK  Function List

Relay_On   X 			   X: 1-4,8

Relay_Off  X 			   X: 1-4,8

GetRelayStatus  X  		   X: 1-4,8

GetAllRelayStatus

StoreDefault

GetDefault

TurnAllRelayOff

TurnAllRelayOn

InvertAllRelay

ReverseRelayOrder

Test2Way

SetAllRelays   X    			X: 0-255

StorePatternInBank  X    		X: 0-15

RecallPatternInBank  X   	    X: 0-15

RelaySelect   X 			    X: 1-4,8

RelayDeselect   X   		    X: 1-4,8

ToggleRelay     X   			X: 1-4,8

SetRelayTimer   X1  X2   [X3] 	      X1 : 0-32000ms, X2 : 0-7, [X3 : 0 or 1]

SetMultiRelaysTimer X1  X2  X3  [X4]	  X1: 0-32000ms, X2, X3 : 0-255   [X4 : 0 or 1]

ReportModeOn

ReportModeOff

ProgramEMUDevNum  X      X: 0-15

Relay_On   X    		X: 1-4,8

Turn on individual relays.

Example

Turn relay 4 on.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  Relay_On  4

Relay_Off   X    		X: 1-4,8

Turn off individual relays.

Example

Turn relay 2 off.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  Relay_Off  2

GetRelayStatus   X    	X: 1-4,8

This command allows you to read the on/off status of an individual relay. This command will return a 1 indicating the relay is ON or a 0 indicating the relay is OFF.

Example

Get the status of relay 3.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK GetRelayStatus  3

GetAllRelayStatus

This command allows you to get the status all relays at one time. A value of 0-255 is returned indicating the status of all 8 relays. The binary pattern of the value returned directly corresponds to the on/off status of each relay.

Example

Get the status of all relays.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK GetAllRelayStatus

StoreDefault

This command allows you to define the on/off status of all relays when power is first applied to the board. Use other commands to set the relays in the desired power-up state, then issue this command to store the current status of the relays as the power-up default.

Example

Store the current relays status as default.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  StoreDefault

GetDefault

This command allows you read the stored power-up default relay pattern. The binary pattern of the value returned directly corresponds to the on/off status of each relay.

Example

Get the default status of relays.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  GetDefault

TurnAllRelayOff

Turn all relays off.

Example

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  TurnAllRelayOff

TurnAllRelayOn

Turn all relays on.

Example

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  TurnAllRelayOn

InvertAllRelay

Toggle the status of a relay.

Example

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  InvertAllRelay

ReverseRelayOrder

The Status of Relays 12345678 are reversed to 87654321. This command does not permanently reassign relays, it only copies the status of the relays when executed.

Example

Reverse the relays' status.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  ReverseRelayOrder

Test2Way

This command can be used to test 2-way communication between the host computer and the relay controller. This command should be used for initial installations if 2-way communication is required. It can also be used to detect the presence of a relay controller on the serial port.

Example

Test 2-Way communication.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  Test2Way

SetAllRelays  X    			X: 0-255

This command is used to set the status of all relays at one time. A single parameter is required. The equivalent binary pattern of the parameter is copied directly to the relays, instantly setting the on/off status of all relays on the board.

Example

Set the status of all relays.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  SetAllRelays  27

StorePatternInBank  X    		X: 0-15

This command stores the current on/off setting of all relays into a memory bank (0-15). This command is useful for creating macros or for making sure certain relays are never activated simultaneously.

Example

Store Relay Pattern in Memory Bank 9.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  StorePatternInBank  9

RecallPatternInBank  X    	X: 0-15

This command recalls a stored relay pattern from the user selected memory bank (0-15) and update all relays on the board to the settings defined by StorePatternInBank.

Example

Recall relay pattern from memory bank 11.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  RecallPatternInBank  11

RelaySelect  X    			X: 1-4,8

This command turns off all relays and then turns on the selected relay only. This command performs a safe “Break Before Make”, ensuring that no two relays are ever activated at the same time.

Example

Turn on relay 3 only.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  RelaySelect  3

RelayDeselect  X    			X: 1-4,8

This command turns on all relays and then turns off the selected relay only. This command performs a safe “Make Before Brake”, ensuring that no two relays are ever de-activated at the same time.

Example

Turn off relay 2 only.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  RelayDeselect  2

ToggleRelay   X      		X: 1-4,8

This command reverses the current on/off status of the selected relay.

Example

Revers the current status of the selected relay 4.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  ToggleRelay  2

SetRelayTimer  X1  X2   [X3] 		  X1 : 0-32000ms, X2 : 0-7, [X3 : 0 or 1]

This command is used to activate a relay for a user-defined period of time. All other relays will remain unchanged. If the selected relay is already on, this function will have no effect, so make sure the relay is off before using this command. This command will send ASCII character code 85 back to the host computer if feedback is enabled.

Example

Activate the relay 5 after 1000 ms.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  SetRelayTimer  1000  5

SetMultiRelaysTimer  X1  X2  X3  [X4]	  		X1: 0-32000ms, X2, X3 : 0-255   [X4 : 0 or 1]

This command is used set the status of all relays (RPOn), apply a timer (Time 0-255), and then set all Relays to a new state once the timer has completed (RPOff). This command will send ASCII character code 85 back to the host computer if the feedback is enabled.

Example

Set all relays 110, and after 2000 ms, set all relays 235.

C:\> ncd.exe  –d  IOADR8X.RELAYBANK  SetMultiRelaysTimer  2000  110  235

ReportModeOn

ReportModeOff

ProgramEMUDevNum  X    	    X: 0-15

# IOADR16X  Function List

Test2Way

SetDefault

GetAnalog8   In1    		In1 : 0-4

GetAnalog10  In1    		In1 : 0-4

SetRelayBankA

SetRelayBankB

SetAll16   In1  In2       In1 : 0-255  In2 : 0-255

GetAll16   In1  In2       In1 : 0-255  In2 : 0-255

Test2Way

This method can be used to test 2-way communication between the host computer and the relay controller.

Example

Test 2-way communication, true for 2-way communication.

C:\> ncd.exe  –d  IOADR16X  Test2Way

SetDefault

This command can be used to restore all configuration parameters to factory default settings. DO NOT ISSUE THIS COMMAND IF MORE THAN ONE HYBRID OR IOAD168 CONTROLLER IS ATTACHED TO THE SAME SERIAL PORT OR ALL CONTROLLERS WILL BE PROGRAMMED WITH FACTORY DEFAULT VALUES.

Example

Restore all configuration to factory default setting.

C:\> ncd.exe  –d  IOADR16X  SetDefault

GetAnalog8   In1            In1 : 0-4

This command requests the analog data value from the user-specified A/D input, and reports back an analog value of 0-255.

Example

Get 8-bits analog value of channel 2.

C:\> ncd.exe  –d  IOADR16X  GetAnalog8  2

GetAnalog10  In1    			In1 : 0-4

This command requests the analog data value from the user-specified A/D input, and reports back an analog value of 0-255.

Example

Get 10-bits analog value of channel 3.

C:\> ncd.exe  –d  IOADR16X  GetAnalog10  3

SetRelayBankA

Direct commands to relay bank A.

Example

C:\> ncd.exe  –d  IOADR16X  SetRelayBankA

SetRelayBankB

Direct commands to relay bank B.

Example

C:\> ncd.exe  –d  IOADR16X  SetRelayBankB

SetAll16   In1  In2    		In1 : 0-255  In2 : 0-255

This Command is used to set the Status of ALL 16 relays at one time. This command requires two parameters. The first parameter sets the status of Relay Bank A, the second parameter sets the status of Relay Bank B. Both Parameters have a valid range of 0-255. Relays will be activated in the equivalent binary pattern of the parameter value.

Example

Set the status of All 16 relays,150 for BankA, 170 for BankB.

C:\> ncd.exe  –d  IOADR16X  SetAll16   150   170

GetAll16

This Command is used to read the Status of ALL 16 relays at one time. This command sends two bytes of data back to the host computer. The first byte of data returned indicates the status of Bank A relays. The second byte of data returned indicates the status of Bank B relays. Each byte of data returned to the host computer has a valid range of 0-255. The binary pattern of the returned valued indicates the on/off status of each relay. The Least Significant Bit of Each Returned Value indicates the Status of Relay 1 on Bank A and Bank B Respectively.

Example

Get the status of all 16 relays.

C:\> ncd.exe  –d  IOADR16X  GetAll16

SUB DEVICE LIST:

LCD

PORTA

PORTB

PORTC

RELAYBANK

KEYBOARD

SUB_LCD  Function List

Init

SendData  In1    		In1 : 0-255

SendCommand   In1    	In1 : 0-255

SendBuffer   In1    		In1 : Text

BacklightOff

BacklightOn

SetPort   In1    			In1 : 1-3

ClearScreen

HomeCursor

BlankDisplay

ToggleCursorVisible

UnderlineCursor

BlockCursor

MoveCursorLeft

MoveCursorRight

SetPointerPosition   In1  	 	 	In1 : 0-63

SetCursorPosition   In1    		In1 : 0-127

SUB_PORTA  Function List

GetBitsStatus   In1        	In1 : 0-7

PutByte   In1        		In1 : 0-255

TurnOnPortBits   In1        	In1 : 0-7

TurnOffPortBits   In1        	In1 : 0-7

ReadPortByte

PAR24  In1  In2    			In1 : 1-6  In2 : 0-255

Ad1216   In1  In2    		In1 : 0-2  In2 : 0-15

SUB_PORTB  Function List

GetBitsStatus   In1        	In1 : 0-7

PutByte   In1        		In1 : 0-255

TurnOnPortBits   In1         In1 : 0-7

TurnOffPortBits   In1         In1 : 0-7

ReadPortByte

PAR24   In1  In2    		In1 : 1-6  In2 : 0-255

Ad1216   In1  In2    		In1 : 0-2  In2 : 0-15

SUB_PORTC  Function List

GetBitsStatus   In1	        In1 : 0-7

PutByte   In1        		In1 : 0-255

TurnOnPortBits   In1        	In1 : 0-7

TurnOffPortBits   In1        	In1 : 0-7

ReadPortByte

SUB_RELAYBANK  Function List

Relay_On   X    			X: 1-4,8

Relay_Off   X    			X: 1-4,8

GetRelayStatus  X    			X: 1-4,8

GetAllRelayStatus

StoreDefault

GetDefault

TurnAllRelayOff

TurnAllRelayOn

InvertAllRelay

ReverseRelayOrder

Test2Way

SetAllRelays  X    			X: 0-255

StorePatternInBank  X    		X: 0-15

RecallPatternInBank  X        X: 0-15

RelaySelect  X  				X: 1-4,8

RelayDeselect  X    			X: 1-4,8

ToggleRelay  X    			X: 1-4,8

SetRelayTimer  X1  X2   [X3] 	  	  X1 : 0-32000ms, X2 : 0-7, [X3 : 0 or 1]

SetMultiRelaysTimer X1  X2  X3  [X4]	  X1: 0-32000ms, X2, X3 : 0-255   [X4 : 0 or 1]

ReportModeOn

ReportModeOff

ProgramEMUDevNum  X     X: 0-15

SUB_KEYBOARD  Function List

Keypad  In1 				   In1 : 0-127

KeyToScreen   In1    	   In1 : 0-127

KeyToScreenHidden  In1      In1 : 0-127

The usage of sub-device’s functions of IOADR16X is same as that of IOADR8X.

# IOCONTROLLER  Function List

WriteByte    In1       		In1 : 0-255

ReadByte

SetBit      In1  In2     		In1 : 0-7    In2 : 0 or 1

GetBit      In1        	 	In1 : 0-7

PAR24     In1  In2    	 	In1 : 1-6    In2 : 0-255

Ad1216    In1  In2     		In1 : 0-2    In2 : 0-15

WriteByte   In1      	   		In1 : 0-255

This command sets the 8-bit data bus to outputs and writes a byte to the data port. This command is very useful for serial-toparallel conversion.

Example

Write data to port 9.

c:\>NCD.exe  -d  IOCONTROLLER  WriteByte   9

ReadByte

This command sets the 8-bit data bus to inputs, reads the status of the port, and reports back a numeric value from 0-255. This command is very useful for parallel-to-serial encoding.

Example

Read port data.

c:\>NCD.exe  -d  IOCONTROLLER  ReadByte

SetBit   In1  In2    			 In1 : 0-7    In2 : 0 or 1

This command is used to turn on an individual data bit on the 8-bit data bus. If the bit is an input, it will automatically become an output and the on/off status will be set. This command has two parameters. Bits, 0-7, etermines which bit is to be altered. Status 0 or 1 determines the on/off status of the selected bit. 0 for off, 1 for on.

Example

Set the status of bit 4 on.

c:\>NCD.exe  -d  IOCONTROLLER  SetBit   4   1

GetBit   In1    			     In1 : 0-7

This command converts the selected bit to an input and reads the on/off status. A value of 0 or 1 is sent back to the computer indicating the status of the bit.

Example

Get the status of bit 2.

c:\>NCD.exe  -d  IOCONTROLLER  GetBit  2

PAR24  In1  In2     		 In1 : 1-6    In2 : 0-255

This command sends a byte of data (0-255) to one of the PAR24 output channels (1-6) on Port. The data appears as binary/ parallel data on the 8-bit output of the PAR24.

Example

Put 17 to channel 3.

c:\>NCD.exe  -d  IOCONTROLLER  PAR24  3  17

Ad1216   In1   In2   	  	In1 : 0-2    In2 : 0-15

This command returns a 12-Bit analog value from Channel 0-15, Defined as Device 0-2 on a AD1216 Connected to IOServo.

Example

Read the analog value from channel 13, device 1.

c:\>NCD.exe  -d  IOCONTROLLER  Ad1216  1  13

# MSERVO  Function List

UpdateAllServos    In1  In2  In3    	In1 - In3 : 0-255

SetServo1Pos       In1      			In1 : 0-255

SetServo2Pos       In1      			In1 : 0-255

SetServo3Pos       In1      			In1 : 0-255

StorePowerupSetting

UpdateAllServos    In1  In2  In3    	In1 - In3 : 0-255

This command is used to program the position of all three servo motors at one time. This command requires 3 parameters indicating the position of each of the three servo motors and a termination code of 85, used to force an immediate servo motor update.

Example

Program the position of all three servo motors at one time.

c:\>NCD.exe  -d  MSERVO  UpdateAllServos  136   168   79

SetServo1Pos    In1  		    		In1 : 0-255

This command positions Servo Motor 1, and does not affect the other motors.

Example

Set the position of Servo Motor 1

c:\>NCD.exe  -d  MSERVO  SetServo1Pos  110

SetServo2Pos    In1       			In1 : 0-255

This command positions Servo Motor 2, and does not affect the other motors.

Example

Set the position of Servo Motor 2

c:\>NCD.exe  -d  MSERVO  SetServo2Pos  150

SetServo3Pos    In1   	   			In1 : 0-255

This command positions Servo Motor 3, and does not affect the other motors.

Example

Set the position of Servo Motor 3

c:\>NCD.exe  -d  MSERVO  SetServo3Pos  180

StorePowerupSetting

This command store the current position of the motors as the power-up default position. The next time power is applied, all three motors will return to this programmed position.

Example

Store the current the position of motors as the power-up default position.

c:\>NCD.exe  -d  MSERVO  StorePowerupSetting

# PWM8X  Function List

SetAllChannels   In1   			  In1 : 1-8

SetChannel       In1  In2  		  In1 : 1-8   In2 : 0-255

StoreStartupPWMValues

StoreStartupMode   In1   			  In1 : 1, not 1

SetAllChannels   In1   	                  In1 : 1-8

Set the value for all channels.

Example

Set all 8 channels with value 1.

c:\>NCD.exe  -d  PWM8X  SetAllChannels  1

SetChannel    In1  In2  				  In1 : 1-8   In2 : 0-255

Set the value for selected channel.

Example

Set channel 4 with value 18.

c:\>NCD.exe  -d  PWM8X  SetChannel   4  18

StoreStartupPWMValues

This command is used to store the currently selected PWM Values for all 8 channels as the default power-up values. This command can be used in a room with DC lighting, allowing different areas of the room to have different levels of brightness when the lights are turned on.

Example

Store the current values of PWM as default power-up values.

c:\>NCD.exe  -d  PWM8X  StoreStartupPWMValues

StoreStartupMode    In1   			 	 In1 : 1, not 1

The Store Startup Mode command is used to set the way the PWM8x powers up. A parameter value of 1 causes all 8 channels to begin cycling immediately, meaning all outputs become immediately active (based on the Stored Default Powerup PWM Values above). A parameter value of anything other than 1 causes each channel to rise slowly to its Stored Default Powerup PWM Value, beginning with output channel 1.

Example

Set all 8 channels to begin cycline immediately.

c:\>NCD.exe  -d  PWM8X  StoreStartupMode  1

# R16X  Function List

Relay_On    X   			 	X: 1-16

Relay_Off    X    				X: 1-16

AllRelaysOn

AllRelaysOff

BankDisable

BankEnable

ClearDefaultRelaySettings

GetAllRelayStatus

GetLeftBankStatus

GetRelayStatus    X   		   	X: 1-16

GetRightBankStatus

LeftBankOff

LeftBankOn

RecallRelaySettings  X  		    X: 0-255

ReportingOff

ReportingOn

RightBankOff

RightBankOn

SetAllRelayStatus   X1  X2  		X1,X2 : 0-255

SetLeftBankStatus   X  			X: 0-255

SetRightBankStatus   X 			X: 0-255

StoreDefaultRelaySettings

StoreRelaySettings    X 		    X: 0-255

StoreReportingStatus

Relay_On    X   			 X: 1-16

Turing On Individual Relays.

Example

Turn Relay 12 on.

c:\>NCD.exe  -d  R16X  Relay_On  12

Relay_Off    X    		     X: 1-16

Turing Off Individual Relays.

Example

Turn Relay 10 off.

c:\>NCD.exe  -d  R16X  Relay_Off  10

AllRelaysOn

Turn on all relays

Example

c:\>NCD.exe  -d  R16X  AllRelaysOn

AllRelaysOff

Turn off all relays

Example

c:\>NCD.exe  -d  R16X  AllRelaysOff

BankDisable

Low power mode allows you to minimize power consumption and may be used as an emergency deactivation of relays. While in low power mode, all relays and LEDs are turned off. The CPU stays running while in low power mode. You will be able to send and receive commands to the relay board as if the board were active. Any change in relay status will not be visible until normal operation is resumed.

Example

Turn the device into low power mode

c:\>NCD.exe  -d  R16X  BankDisable

BankEnable

Resumes normal operation. The status of relays may change when in low power mode. The status will not be reflected on the LED and relays until this command is executed.

Example

Resumes normal operation

c:\>NCD.exe  -d  R16X  BankEnable

ClearDefaultRelaySettings

This command erases the stored power-up status. All relays will be off when power is first applied.

Example

Erase the stored power-up status.

c:\>NCD.exe  -d  R16X  ClearDefaultRelaySettings

GetAllRelayStatus

Ask the R16 for the status of all relays.

Example

Get the status of all relays.

c:\>NCD.exe  -d  R16X  GetAllRelayStatus

GetLeftBankStatus

Ask the R16 for the status of the left relay bank. A byte will be reported back to the computer indicating the status of all 8 leftbank relays.

Example

Get the status of left bank

c:\>NCD.exe  -d  R16X  GetLeftBankStatus

GetRelayStatus    X   			X: 1-16

Ask the R16 for the status of relay 1-16. The R16 will report back a status for the selected relay. A status reply of 0 indicates the relay is off. A status reply of 1 indicates the relay is on.

Example

Get the status of relay 7.

c:\>NCD.exe  -d  R16X  GetRelayStatus  7

GetRightBankStatus

Ask the R16 for the status of the right relay bank. A byte will be reported back to the computer indicating the status of all 8 rightbank relays.

Example

Get the status of right bank.

c:\>NCD.exe  -d  R16X  GetRelayStatus  7

LeftBankOff

Turn off left bank relays.

Example

c:\>NCD.exe  -d  R16X  LeftBankOff

LeftBankOn

Turn on left bank relays.

Example

c:\>NCD.exe  -d  R16X  LeftBankOn

RecallRelaySettings   X  		   X: 0-255

Recalls the on/off status of all 16 relays from 1 of 256 different memory banks. A parameter is required for this command indicating the memory bank to recall.

Example

Recall the current relays status from memory bank 36.

c:\>NCD.exe  -d  R16X  RecallRelaySettings  36

ReportingOff

The R16 does NOT send ASCII 85 to acknowledge incoming commands. Provides faster, but blind operation.

When reporting mode is turned on, the R16 acknowledges most incoming commands by sending ASCII character code 85 back to the computer once the command has been executed by the board. When properly used, this can prevent you from sending commands to the R16 faster than they can be processed. And because incoming commands are acknowledged, you know the board is properly functioning from your desktop computer.

Example

Turn report mode off.

c:\>NCD.exe  -d  R16X  ReportingOff

ReportingOn

The R16 sends ASCII 85 back to the computer after most commands to acknowledge execution. Slower, but the receipt of commands is confirmed by the R16.

When reporting mode is turned on, the R16 acknowledges most incoming commands by sending ASCII character code 85 back to the computer once the command has been executed by the board. When properly used, this can prevent you from sending commands to the R16 faster than they can be processed. And because incoming commands are acknowledged, you know the board is properly functioning from your desktop computer.

Example

Turn on the report mode

c:\>NCD.exe  -d  R16X  ReportingOn

RightBankOff

Turn off right bank relays.

Example

c:\>NCD.exe  -d  R16X  RightBankOff

RightBankOn

Turn on right bank relays.

Example

c:\>NCD.exe  -d  R16X  RightBankOn

SetAllRelayStatus    X1  X2  			X1,X2 : 0-255

This command allows you to set the status of all relays at one time. Left bank relays will be activated in the equivalent binary pattern of the first parameter byte. Right bank relays will be activated in the equivalent binary pattern of the second parameter byte.

Example

Set the status of left bank relays to 80, set the status of right bank relays to 100.

c:\>NCD.exe  -d  R16X  SetAllRelayStatus  80  100

SetLeftBankStatus    X  			 X: 0-255

This command allows you to set the status of all left bank relays at one time. Relays will be activated in the equivalent binary pattern of the parameter byte.

Example

Set the status of left bank relays to 166.

c:\>NCD.exe  -d  R16X  SetLeftBankStatus  166

SetRightBankStatus   X 			 X: 0-255

This command allows you to set the status of all right bank relays at one time. Relays will be activated in the equivalent binary pattern of the parameter byte.

Example

Set the status of right bank relays to 220.

c:\>NCD.exe  -d  R16X  SetRightBankStatus  220

StoreDefaultRelaySettings

It is possible to control the power-up status of relays. By default, all relays are turned off when power is first applied to the board. To use this command, use other relay control commands to set the status of the relays in the desired default state. Then execute this command to record the relay settings. The next time power is applied to the board, the relays will automatically return to the stored power-up status.

Example

Set the current status of relays as default setting.

c:\>NCD.exe  -d  R16X  StoreDefaultRelaySettings

StoreRelaySettings    X 		      X: 0-255

The R16 allows you to store and recall the status of all 16 relays in 256 different memory banks. This is useful for creating hardware macros that electronically prevent some relays from coming on simultaneously. To use this command, use other relay control commands to set the status of all 16 relays. Once relays are set, the status can be stored. A parameter is required for this command, indicating the memory bank to store the relay setting in.

Example

Save the current relays status to memory bank 48.

c:\>NCD.exe  -d  R16X  StoreRelaySettings  48

StoreReportingStatus

Once you have set the reporting mode, you can store this mode as the default. The next time power is applied to the board, the reporting mode will be recalled from stored memory. The R16 is shipped with the reporting mode turned on. The R16 setup utility turns on the reporting mode, but does not change the stored settings.

Example

Store the current report mode as default.

c:\>NCD.exe  -d  R16X  StoreReportingStatus

# R1X  Function List

Relay_On

Relay_Off

GetRelayStatus

StorePowerDefault

GetStartupDefault

Relay_On

Turn on the relay.

Example

c:\>NCD.exe  -d  R1X  Relay_On

Relay_Off

Turn off the relay.

Example

c:\>NCD.exe  -d  R1X  Relay_Off

GetRelayStatus

This Command reports back 0 or 1 indicating the status of Relay. 0 = Off, 1 = On.

Example

c:\>NCD.exe  -d  R1X  GetRelayStatus

StorePowerDefault

This command will store the current on/off state of the relays in non-volatile EEPROM. The next time power is applied, the relays will automatically return to the store on/off state.

Example

Store the current on/off  state of the relays in non-volatile EEPROM.

c:\>NCD.exe  -d  R1X  StorePowerDefault

GetStartupDefault

This Command reports back 0 or 1 indicating the startup default status of Relay. 0 = Off, 1 = On.

Example

Get relay startup status.

c:\>NCD.exe  -d  R1X  GetStartupDefault

# R2X  Function List

Relay_On    X    		 	 X: 1 or 2

Relay_Off    X   			 X: 1 or 2

GetRelayStatus  X    			 X: 1 or 2

GetBothRelayStatus

SetBothRelayStatus  X    		 X: 0-3

StorePowerDefault

GetStartupDefault

Relay_On    X    			X: 1 or 2

Turn on individual relays.

Example

Turn relay 2 on.

c:\>NCD.exe  -d  R16X  Relay_On  2

Relay_Off    X   			 X: 1 or 2

Turn off individual relays.

Example

Turn relay 2 off.

c:\>NCD.exe  -d  R16X  Relay_Off  2

GetRelayStatus  X    			 X: 1 or 2

This command allows you to read the on/off status of an individual relay. This command will return a 1 indicating the relay is ON or a 0 indicating the relay is OFF.

Example

Get the status of realy 1.

c:\>NCD.exe  -d  R16X  GetRelayStatus  1

GetBothRelayStatus

This command reports the on/off status of both relays. This command will send a byte from 0 to 3 back to the user indicating the status of both relays:

Return Byte 0: Both Relays are Off

Return Byte 1: Relay 1 is On, Relay 2 is Off

Return Byte 2: Relay 2 is On, Relay 1 is Off

Return Byte 3: Both Relays are On

Example

Get the status of both relays.

c:\>NCD.exe  -d  R16X  GetBothRelayStatus

SetBothRelayStatus  X    		  X: 0-3

This command sets the status of both relays at one time. This command requires a parameter of 0-3:

Parameter 0: Turn Off Both Relays

Parameter 1: Turn On Relay 1, Turn Off Relay 2

Parameter 2: Turn On Relay 2, Turn Off Relay 1

Parameter 3: Turn On Both Relays.

Example

Set the status of both relays.

c:\>NCD.exe  -d  R16X  SetBothRelayStatus  3

StorePowerDefault

This command will store the current on/off state of the relays in non-volatile EEPROM. The next time power is applied, the relays will automatically return to the store on/off state.

Example

Store the current on/off state of the relays in non-volatile EEPROM.

c:\>NCD.exe  -d  R16X  StorePowerDefault

GetStartupDefault

This command retrieves the stored power-up default state of the relays. This command will send a byte from 0-3 indicating the on/off state of the relays when power is first applied to the board.

Return Byte 0: Both Relays are Off on Power-up

Return Byte 1: Relay 1 is On, Relay 2 is Off on Power-up

Return Byte 2: Relay 2 is On, Relay 1 is Off on Power-up

Return Byte 3: Both Relays are On when Powered-up

Example

Get the default the setting of relays.

c:\>NCD.exe  -d  R16X  GetStartupDefault

# R32X  Function List

Relay_On     X    			X: 1-32

Relay_Off     X    			X: 1-32

SetBankStatus  X1  X2  		X1: 1-4 X2: 0 - 255

SetBankA     X    			X: 0-255

SetBankB     X    			X: 0-255

SetBankC     X    			X: 0-255

SetBankD     X    			X: 0-255

SetBankAB    X1  X2  		X1,X2 : 0-255

SetBankCD    X1  X2  		X1,X2 : 0-255

SetBankAll     X1  X2  X3  X4  		X1,X2,X3,X4 : 0-255

AllRelaysOff

AllRelaysOn

GetRelayStatus  X   			 X: 1-32

GetBankStatus   X  			 X: 1-4

GetAllBankStatus

GetReportingMode

StorePowerupDefault

StoreRelaySettings   X   		 X: 0-255

RecallRelaySettings  X  		 X: 0-255

Relay_On   X    			X: 1-32

Turn on individual relays.

Example

Turn on relay 15.

c:\>NCD.exe  -d  R32X  Relay_On   15

Relay_Off   X    				X: 1-32

Turn off individual relays.

Example

Turn off relay 19.

c:\>NCD.exe  -d  R32X  Relay_Off   19

SetBankStatus  X1  X2  			 X1: 1-4    X2: 0 – 255

This command allows you to set the status of selected Bank relays at one time. The parameter of status is required for this command, indicating the status of all 8 relays. Relays will be activated in the equivalent binary pattern of the parameter byte.

Example

Set the Bank D relays all on.

c:\>NCD.exe  -d  R32X  SetBankStatus   4  255

SetBankA  X    				X: 0-255

This command allows you to set the status all Bank A relays at one time. A parameter of 0-255 is required for this command, indicating the status of all 8 relays. Relays will be activated in the equivalent binary pattern of the parameter byte.

Example

Set the status all Bank A relays at one time.

c:\>NCD.exe  -d  R32X  SetBankA   115

SetBankB  X    				X: 0-255

This command allows you to set the status all Bank B relays at one time. A parameter of 0-255 is required for this command, indicating the status of all 8 relays. Relays will be activated in the equivalent binary pattern of the parameter byte.

Example

Set the status all Bank B relays at one time.

c:\>NCD.exe  -d  R32X  SetBankB   65

SetBankC  X    				X: 0-255

This command allows you to set the status all Bank C relays at one time. A parameter of 0-255 is required for this command, indicating the status of all 8 relays. Relays will be activated in the equivalent binary pattern of the parameter byte.

Example

Set the status all Bank C relays at one time.

c:\>NCD.exe  -d  R32X  SetBankC   125

SetBankD  X    				X: 0-255

This command allows you to set the status all Bank D relays at one time. A parameter of 0-255 is required for this command, indicating the status of all 8 relays. Relays will be activated in the equivalent binary pattern of the parameter byte.

Example

Set the status all Bank D relays at one time.

c:\>NCD.exe  -d  R32X  SetBankD   125

SetBankAB  X1  X2  				X1,X2 : 0-255

This command allows you to set the status all relays in Bank A and Bank B. This command requires two parameters, each with a valid value of 0-255. The first parameter will set the status of Bank A relays in the equivalent binary pattern. The second parameter will set the status of Bank B relays in the equivalent binary pattern.

Example

Set the status all relays in Bank A and Bank B.

c:\>NCD.exe  -d  R32X  SetBankAB   60  140

SetBankCD  X1  X2  				X1,X2 : 0-255

This command allows you to set the status all relays in Bank C and Bank D. This command requires two parameters, each with a valid value of 0-255. The first parameter will set the status of Bank C relays in the equivalent binary pattern. The second parameter will set the status of Bank D relays in the equivalent binary pattern.

Example

Set the status all relays in Bank C and Bank D.

c:\>NCD.exe  -d  R32X  SetBankCD   25  78

SetBankAll  X1  X2  X3  X4  		X1,X2,X3,X4 : 0-255

This command sets the status of all 32 relays. It requires 4 parameters, indicating the binary status of banks A, B, C, and D.

Example

Set the status of all 32 relays.

c:\>NCD.exe  -d  R32X  SetBankAll   38  146  92  220

AllRelaysOff

Turn off all relays.

Example

c:\>NCD.exe  -d  R32X  AllRelaysOff

AllRelaysOn

Turn on all relays.

Example

c:\>NCD.exe  -d  R32X  AllRelaysOn

GetRelayStatus   X   				 X: 1-32

Ask the R32 for the status of relay 1-32. The R32 will report back a status for the selected relay. A status reply of 0 indicates the relay is off. A status reply of 1 indicates the relay is on.

Example

Get the stauts of relay 19

c:\>NCD.exe  -d  R32X  GetRelayStatus   19

GetBankStatus  X  				  X: 1-4

This command reports the status of a bank of relays back to the user. This command requires a parameter, 1-4, indicating the relay bank, A-D, to be read. This command will report back an ASCII character code from 0 to 255. Relays are set in the equivalent binary pattern of this number.

Example

Get the status of bank B.

c:\>NCD.exe  -d  R32X  GetBankStatus  2

GetAllBankStatus

This command reports the status of all relays back to the user. This command will send 4 bytes back to the user, representing the status of banks A, B, C, and D. Relays are set in the equivalent binary pattern of every number returned.

Example

Get the status of Bank A, B, C, D.

c:\>NCD.exe  -d  R32X  GetAllBankStatus

GetReportingMode

Reads the Reporting Mode status setting (Dip switch 4). When this dip switch is ON, the R32 will confirm the receipt of commands by sending ASCII character code 85.

Example

Get the status of report mode.

c:\>NCD.exe  -d  R32X  GetReportingMode

StorePowerupDefault

This command stores the current status of all 32 relays in non-volatile memory. The next time power is applied to the R32, the relays will return to the stored pattern.

Example

Store the current status of all 32 relays as power up default setting.

c:\>NCD.exe  -d  R32X  StorePowerupDefault

StoreRelaySettings   X   				 X: 0-255

This command stores the current status of all 32 relays into a user-selected memory bank. This command requires a parameter from 0 to 255, indicating the memory bank to store the current relay pattern.

Example

Store the current status of all 32 relays into memery bank 50.

c:\>NCD.exe  -d  R32X  StoreRelaySettings

RecallRelaySettings  X  				  X: 0-255

This command retrieves a relay pattern from the user selected memory bank. This command requires a parameter from 0 to 255, indicating the memory bank to retrieve. All 32 relays will be set according to the data retrieved from memory.

Example

Retrieves a relay pattern from memory bank 40.

c:\>NCD.exe  -d  R32X  RecallRelaySettings  40

# R4X  Function List

Relay_On  X  		    X: 1-4,8

Relay_Off  X    			X: 1-4,8

GetRelayStatus  X    		X: 1-4,8

GetAllRelayStatus

StoreDefault

GetDefault

TurnAllRelayOff

TurnAllRelayOn

InvertAllRelay

ReverseRelayOrder

Test2Way

SetAllRelays  X    		 X: 0-255

StorePatternInBank  X      X: 0-15

RecallPatternInBank  X     X: 0-15

RelaySelect    X  		 X: 1-4,8

RelayDeselect  X 			 X: 1-4,8

ToggleRelay    X 		 X: 1-4,8

SetRelayTimer  X1  X2  [X3]				      X1 : 0-32000ms, X2 : 0-7, [X3 : 0 or 1]

SetMultiRelaysTimer  X1  X2  X3  [X4]		  X1:0-32000ms, X2, X3 : 0-255   [X4 : 0 or 1]

# R8XPRO  Function List

Relay_On     X    		X: 1-4,8

Relay_Off     X    		X: 1-4,8

GetRelayStatus  X    		X: 1-4,8

GetAllRelayStatus

StoreDefault

GetDefault

TurnAllRelayOff

TurnAllRelayOn

InvertAllRelay

ReverseRelayOrder

Test2Way

SetAllRelays   X   	 		X: 0-255

StorePatternInBank  X   		X: 0-15

RecallPatternInBank  X   		X: 0-15

RelaySelect   X    			X: 1-4,8

RelayDeselect  X    			X: 1-4,8

ToggleRelay  X    			X: 1-4,8

SetRelayTimer  X1  X2  [X3]		          X1 : 0-32000ms, X2 : 0-7, [X3 : 0 or 1]

SetMultiRelaysTimer X1  X2  X3  [X4]		  X1:0-32000ms, X2, X3 : 0-255   [X4 : 0 or 1]

ReportModeOn

ReportModeOff

ProgramEMUDevNum  X  		  X: 0-15

Relay_On  X		    		X: 1-4,8

Turn on individual relays.

Example

Turn relay 2 on.

c:\>NCD.exe  -d  R8XPRO  Relay_On  2

Relay_Off   X    		X: 1-4,8

Turn off individual relays.

Example

Turn relay 1 off.

c:\>NCD.exe  -d  R8XPRO  Relay_Off  1

GetRelayStatus  X    		X: 1-4,8

This command allows you to read the on/off status of an individual relay. This command will return a 1 indicating the relay is ON or a 0 indicating the relay is OFF.

Example

Get the status of relay 3.

c:\>NCD.exe  -d  R8XPRO  GetRelayStatus  3

GetAllRelayStatus

This command allows you to get the status all relays at one time. A value of 0-255 is returned indicating the status of all 8 relays. The binary pattern of the value returned directly corresponds to the on/off status of each relay.

Example

Get the status of all relays.

c:\>NCD.exe  -d  R8XPRO  GetAllRelayStatus

StoreDefault

This command allows you to define the on/off status of all relays when power is first applied to the board. Use other commands to set the relays in the desired power-up state, then issue this command to store the current status of the relays as the power-up default.

Example

Store the current relays status as default.

c:\>NCD.exe  -d  R8XPRO  StoreDefault

GetDefault

This command allows you read the stored power-up default relay pattern. The binary pattern of the value returned directly corresponds to the on/off status of each relay.

Example

Get the default status of relays.

c:\>NCD.exe  -d  R8XPRO  GetDefault

TurnAllRelayOff

Turn all relays off.

Example

c:\>NCD.exe  -d  R8XPRO  TurnAllRelayOff

TurnAllRelayOn

Turn all relays on.

Example

c:\>NCD.exe  -d  R8XPRO  TurnAllRelayOn

InvertAllRelay

Toggle the status of a relay.

Example

c:\>NCD.exe  -d  R8XPRO  InvertAllRelay

ReverseRelayOrder

The Status of Relays 12345678 are reversed to 87654321. This command does not permanently reassign relays, it only copies the status of the relays when executed.

Example

Reverse the relays' status.

c:\>NCD.exe  -d  R8XPRO  ReverseRelayOrder

Test2Way

This command can be used to test 2-way communication between the host computer and the relay controller. This command should be used for initial installations if 2-way communication is required. It can also be used to detect the presence of a relay controller on the serial port.

Example

Test 2-Way communication.

c:\>NCD.exe  -d  R8XPRO  Test2Way

SetAllRelays  X 	  	 		X: 0-255

This command is used to set the status of all relays at one time. A single parameter is required. The equivalent binary pattern of the parameter is copied directly to the relays, instantly setting the on/off status of all relays on the board.

Example

Set the status of all relays at one time.

c:\>NCD.exe  -d  R8XPRO  SetAllRelays  100

StorePatternInBank  X   		 X: 0-15

This command stores the current on/off setting of all relays into a memory bank (0-15). This command is useful for creating macros or for making sure certain relays are never activated simultaneously.

Example

Store Relay Pattern in Memory Bank 7.

c:\>NCD.exe  -d  R8XPRO  StorePatternInBank  7

RecallPatternInBank  X   		 X: 0-15

This command recalls a stored relay pattern from the user selected memory bank (0-15) and update all relays on the board to the settings defined by StorePatternInBank.

Example

Recall relay pattern from memory bank 10.

c:\>NCD.exe  -d  R8XPRO  RecallPatternInBank  10

RelaySelect  X    			X: 1-4,8

This command turns off all relays and then turns on the selected relay only. This command performs a safe “Break Before Make”, ensuring that no two relays are ever activated at the same time.

Example

Turn on relay 4 only.

c:\>NCD.exe  -d  R8XPRO  RelaySelect  4

RelayDeselect  X    			X: 1-4,8

This command turns on all relays and then turns off the selected relay only. This command performs a safe “Make Before Brake”, ensuring that no two relays are ever de-activated at the same time.

Example

Turn off relay 3 only.

c:\>NCD.exe  -d  R8XPRO  RelayDeselect  3

ToggleRelay  X    			X: 1-4,8

This command reverses the current on/off status of the selected relay.

Example

Reverses the current status of the selected relay 2.

c:\>NCD.exe  -d  R8XPRO  ToggleRelay  2

SetRelayTimer  X1  X2  [X3]		    			X1 : 0-32000ms, X2 : 0-7, [X3 : 0 or 1]

This command is used to activate a relay for a user-defined period of time. All other relays will remain unchanged. If the selected relay is already on, this function will have no effect, so make sure the relay is off before using this command. This command will send ASCII character code 85 back to the host computer if feedback is enabled.

Example

Activate the relay 6 after 500 ms.

c:\>NCD.exe  -d  R8XPRO  SetRelayTimer  500  6

SetMultiRelaysTimer X1  X2  X3  [X4]		    X1:0-32000ms, X2, X3 : 0-255   [X4 : 0 or 1]

This command is used set the status of all relays (RPOn), apply a timer (Time 0-255), and then set all Relays to a new state once the timer has completed (RPOff). This command will send ASCII character code 85 back to the host computer if the feedback is enabled.

Example

Set all relays off, and after 500 ms, set all relays on.

. 		c:\>NCD.exe  -d  R8XPRO  SetMultiRelaysTimer  500  0   255

ReportModeOn

By default, Reporting Mode is off. Reporting mode confirms the ompletion of most commands by sending an ASCII character code 85 back to the user. Reporting mode does NOT confirm the completion of E3C commands, or any command that sends data back to the user. Reporting mode is stored in non-volatile memory. Reporting mode slows communications and is only recommended in high-reliability installations.

Example

Turn report mode on.

.		c:\>NCD.exe  -d  R8XPRO  ReportModeOn

ReportModeOff

This command turns off the reporting function. Reporting mode is stored in non-volatile memory. This is the default setting.

Example

Turn report mode off.

.		c:\>NCD.exe  -d  R8XPRO  ReportModeOff

ProgramEMUDevNum  X    X: 0-15

When the R4x/R8x Pro is NOT in emulation mode, this command can be used, along with its parameter of 0-15, to define the device number for use in emulation mode. Once programmed, you must remove power from the board and set the board to Emulation mode.

Example

Define the device number 13.

.		 c:\>NCD.exe  -d  R8XPRO  ProgramEMUDevNum  13

# SSR8X  Function List

Relay_On  X 	   		X: 1-4,8

Relay_Off  X    			X: 1-4,8

GetRelayStatus  X 	   	X: 1-4,8

GetAllRelayStatus

StoreDefault

GetDefault

TurnAllRelayOff

TurnAllRelayOn

InvertAllRelay

ReverseRelayOrder

Test2Way

SetAllRelays  X    			X: 0-255

StorePatternInBank  X    		X: 0-15

RecallPatternInBank  X    	X: 0-15

RelaySelect  X    			X: 1-4,8

RelayDeselect  X    			X: 1-4,8

ToggleRelay   X    			X: 1-4,8

SetRelayTimer  X1  X2  [X3]  	   		  X1 : 0-32000ms, X2 : 0-7, [X3 : 0 or 1]

SetMultiRelaysTimer  X1  X2  X3  [X4]	  X1:0-32000ms, X2, X3 : 0-255   [X4 : 0 or 1]

ReportModeOn

ReportModeOff

ProgramEMUDevNum  X  		 X: 0-15

The usage of above functions is same as that of R8XPRO.

# SERVOCONTROLLER  Function List

Test2Way

ServoPosition      In1  In2    		  In1 : 1-8    In2 : 0-255

GetServoPosition   In1       		 	  In1 : 1-8

StoreStartupPos

SetAllPos    In1  In2  In3  In4  In5  In6  In7  In8 	  	    In1 - In8: 1-8

CenterAllServos

ConfigureServo    In1  In2  In3  [In4]		      In1 : 0-255    In2 : 20-255

In3 : 20-40    [In4] : 0 or 1

Test2Way

This command is use to test communication between the IOServo and the computer controlling it.

Example

Test 2-Way communication.

c:\>NCD.exe  -d  SERVOCONTROLLER  Test2Way

ServoPosition   In1  In2    			 In1 : 1-8    In2 : 0-255

This command is used to set the position of each individual servo motor. This command requires a parameter of 0-255 indicating the position the servo motor will moved to.

Example

Set the motor 5's position to 128.

c:\>NCD.exe  -d  SERVOCONTROLLER  ServoPosition  5  128

GetServoPosition   In1       			  In1 : 1-8

This command are used to retrieve the position of a servo motor. A value of 0-255 will be sent from the IOServo back to the computer indicating the current position of the servo.

Example

Get the position of motor 6.

c:\>NCD.exe  -d  SERVOCONTROLLER  GetServoPosition  6

StoreStartupPos

This command stores the current position of all servos in nonvolatile EEPROM memory. The next time power is applied to the IOServo, the motors will automatically move to this position. If you are using the Quick Start Kit, you can test this command by simply pressing the RESET button on the RSIO board. This command will report back ASCII code 85 when this operation is complete if reporting mode is turned on. A short delay, 1 second maximum, should be used after this command when using 1-way communication to the IOServo.

Example

Stores the current position of all servos in nonvolatile EEPROM memory.

c:\>NCD.exe  -d  SERVOCONTROLLER  StoreStartupPos

SetAllPos    In1  In2  In3  In4  In5  In6  In7  In8 	  	    In1 - In8: 1-8

This command is used to set the position of all 8 servo motors simultaneously. This command requires 8 parameters, indicating the position of each servo motor.

Example

Set the position of all motors.

c:\>NCD.exe  -d  SERVOCONTROLLER  SetAllPos  2  5  3  6  2  7  1  8

CenterAllServos

This command is a simple shortcut for homing all servos to the center position. It is a very useful command for calibrating the controller to the user-specified range of motion. When properly calibrated, all servo motors should be centered.

Example

Home all servos to the center position.

c:\>NCD.exe  -d  SERVOCONTROLLER  CenterAllServos

ConfigureServo   In1  In2  In3  [In4]			      In1 : 0-255    In2 : 20-255

In3 : 20-40    [In4] : 0 or 1

Calibration setting are stored along with a few other parameters that have nothing to do with calibration, therefore, it is important to understand all parameters for this command before proceeding.

Parameter 1: Base Time 0-255

Base time is used to define the EXACT center of the servo motors you are using. Use the “Center All Servos” to quickly center all servos. Then adjust this parameter until all servos are properly centered. Note that this parameter will need readjustment when the Multiplier value is changed.

Parameter 2: Timeout 20-255

Timeout determines how much time the IOServo spends waiting for a command from the user. The remaining time is spent updating the servo positions. Adjust this parameter if you find the IOServo is not able to catch all of the commands you send to it. A value of 50 works well for this parameter at 9600 baud. Increasing the value spends more time on serial communication. Reducing the value increase the refresh rate of the servos.

Parameter 3: Multiplier 20-40

The Multiplier parameter it what determines the range of motion for the servo. A low value of 20 allows the servo to move in a very narrow range of motion at very high resolution. A value of 40 allows the servo motor to move across a very wide range at a lower resolution. Resolution is determined by the fact that we currently only allow a single byte to determine the position of each servo. Therefore, the range of motion will be divided into 256 parts. A good value for the multiplier can be found by moving the servo across the desired range of motion. Reduce the value until the motor stops jittering when moving to the extreme limits of the motor. The Base time should be used to correct the exact center of the motor once the Multiplier is set. Minor tweaking between these two parameters will be necessary.

Parameter 4: Reply 0-1

By default, Reply is turned off (Reply = 0). If you need the IOServo to confirm the receipt of commands, turn on the Reply mode (Reply = 1). A value of 85 will be returned by all commands that do not already report data back to the computer.

Example

Config the ServoController.

c:\>NCD.exe  -d  SERVOCONTROLLER  ConfigureServo  56  100  28  1

# STP  Function List

SetCoilStatus  In1  In2  In3  In4         In1 - In4 : 0 or 1

SetCoilStatus  In1 In2  In3  In4             n1 - In4 : 0 or 1

Set the Status of Coils.

Example

.		 c:\>NCD.exe  -d  STP  SetCoilStatus  0  1  1  0