-------------------------------

+ Packet Bridge Documentation +

-------------------------------

All right... let's get it straight as quickly as possible.

-----------

+ History +

-----------

Some 6 months ago I had to install a 5 workstation Novell network in a

lab, and then connect it to the University backbone. The first task was

accomplished without too much sweat, but the second one became a real problem.

The rest of Novell networks in University were connected through expensive

hardware bridges, at a price of about $4K each. Our budget was reduced, so we

needed another solution. I tried PC Bridge and several other bridging software,

but none worked with our available hardware. Finally, I got the Packet Driver

Specs and decided to write my own bridge. We have been using it for 6 months

now, and it seems pretty stable (actually, it's the only part of the whole

installation that has not given us any problems in this time).

The performance of the program might be low, but it serves our needs perfectly.

The expected environment for this bridge is the connection of a small to the

central network of the location, in order to get the availabilty of all usual

Internet services (ftp, telnet, etc...). It passes IP and ARP packets around,

so most (if not all) of the Internet protocol based applications will work

perfectly.

As its working is completely interrupt-driven, it TSR after installation, so

the computer can be used even while it is performing as a bridge. Anyway, we

a dedicated PC for this task, and I would not recommend trying to execute

programs as Microsoft Windows while the bridge is installed (I haven't tried,

though). Probably, there won't be too much trouble running programs that

do not enter protected mode.

I provide the source and the executable of the IP/ARP bridge as PKTBRI.ASM and

PKTBRI.COM. There is as well a version that passes IPX packets (the protocol

used by Novell Netware), but it hasn't been tested thoroughly. This second

version is labeled PKTBRIX.

------------------------

+ Use of this software +

------------------------

The installation is extremely simple - just install two network cards on your

intended bridge (a simple PC compatible running MSDOS) and load one Packet

Driver for each card, using interrupt vectors 0x61 and 0x60. Then load the

bridge and 'voila!' you have a nice bridge. We have a dedicated CPU for this

task, with no monitor and keyboard. You can use the computer as usual, though,

as it will stay TSR and return to DOS when installed correctly.

Don't expect it to be too profesional - it's just a 1700 bytes COM! If you

can get it to work, it'll work nicely. If you can't, try to fiddle around with

the source. Compile it with Borland Turbo Assembler (TASM).

---------------------------

+ Status of this software +

---------------------------

This software is PUBLIC DOMAIN, FREEWARE, or however you want to call it. You

can do with it whatever you want, or use it anywhere.

There are two things, though, I'd like to be clear:

- Do not charge anyone for it. I'm not charging you anything, and it's me who

did the actual work.

- Do not claim that it is yours. If you develop something based on this

(significant changes), then it's your program. Just remind you that etiquette

tells that you should acknowledge me.

Anyway, I won't sue you for selling it and earning zillions of dollars - I am

just a student and I don't have the means to do it. You should be congratulated

if you achieve this - that's commercial skill!.

You have the source, it's messy (I know), but look at the header: I did all

the coding in less than a week (from scratch to the whole thing), so you know.

-----------------

+ Contacting me +

-----------------

In case you'd like to tell me anything, here are my addresses:

Snailmail:          Jon Beltran de Heredia

A. M. de Oviedo 3, 1

01007 Gasteiz - Araba

SPAIN

Internet:           jtabeolj@bi.ehu.es

FidoNet:            2:343/121.8

The best ways are Internet and Fido - I *might* answer a snailmail letter, but

I will surely answer a Internet email or Fido message.

--------------------

+ Acknowledgements +

--------------------

This work has been done with the help of the Signal and Communications Group

of the University of the Basque Country.

Thanks to the ETSIT Speech Lab people.

Special thanks to Inma Hernaez for everything.

--------------------------------------------------------------------------------

Signed Jon Beltran de Heredia,                       Bilbao, September 28th,1994